| Host-Name | IP | Interface | pings | gateway |
| --- | --- | --- | --- | --- |
| Router-WAN | ETH4: 172.31.0.1/24 ETH5: | Eth4- nest06a1 Eth5- maindhcp | Firewall Security Desktop |  |
| Security Desktop | 172.31.0.100 | nest06a1 | Router-WAN | 172.31.01 |
| Firewall | WAN(vtnet0):172.31.0.2 DMZ(vtnet1):10.0.1.1 LAN(vtnet2):192.168.2.1 | nest06a1 nest06a2 nest06a3 - | Router-WAN Router-LAN WebServer Databases |  |
| WebServer | 10.0.1.100 255.255.255.0 10.0.1.1 | Nest06a2 - | Firewall | 10.0.1.1 |
| Databases | 10.0.1.200 255.255.255.0 10.0.1.1 | Nest06a2 - | Firewall | 10.0.1.1 |
| Router-LAN | ETH4: 192.168.2.2/24 ETH5: 192.168.1.1/24 | ETH4 - Nest06a3 ETH5 - nest06a4 | Firewall AD/DNS Client-WIN11 Client-LINUX |  |
| AD/DNS | 192.168.1.5 255.255.255.0 192.168.1.1 | Nest06a4 - | Router-LAN | 192.168.1.1 |
| Client-WIN11 | 192.168.1.100 255.255.255.0 192.168.1.1 | Nest06a4 - | Router-LAN | 192.168.1.1 |
| Client-LINUX | 192.168.1.101 255.255.255.0 192.168.1.1 | Nest06a4 - | Router-LAN | 192.168.1.1 |

#####

I don't know what the tracker VM is

I set the IPs, submasks, gateways, and DNS on : Linux client, DNS, Database, webserver, Security desk, RWAN, and RLAN

I reset the firewall so you guys can get a fresh start and try to configure and see if you can connect the 3 networks through it

I highlighted in green what device can ping to what device. It doesnt go both ways due to firewall rules on DNS pc

I also highlighted the interfaces so you can see what the bridge connections are

#####

How to Configure vyos:

Configure

Show interfaces

set interfaces ethernet eth4 address 'X.X.X.X/xx' 	//set ip to interface

Delete interfaces ethernet eth4 address 'X.X.X.X/xx' 	// delete interface configuration

Commit	 // commits the change

Save		// saves to console

Show interface