OPNsense Firewall & Network

Fix Guide for Jason

Based on the March 24 audit. Follow these steps in order.

WARNING: SSH is working to all 8 lab machines right now. Do NOT break it. Read this section before touching anything.

We spent all of Session 10 getting SSH key auth deployed from Kali to every machine. The firewall rules you add must keep SSH traffic flowing. Here is exactly what needs to stay working:

| SSH Connection | Target IP | User | Special Flags |
| --- | --- | --- | --- |
| Kali → OPNsense | 172.31.0.2 | root | -o HostKeyAlgorithms=+ssh-rsa |
| Kali → Router-WAN | 172.31.0.1 | vyos | -o HostKeyAlgorithms=+ssh-rsa |
| Kali → WebServer | 10.0.1.100 | Administrator | (none) |
| Kali → Database | 10.0.1.200 | playerone | (none) |
| Kali → Router-LAN | 192.168.1.1 | vyos | -o HostKeyAlgorithms=+ssh-rsa |
| Kali → AD/DNS | 192.168.1.5 | Administrator | (none) |
| Kali → Client-WIN11 | 192.168.1.100 | Administrator | (none) |
| Kali → Client-LINUX | 192.168.1.51 | playerone | (none) |

All SSH connections originate from Kali at 172.31.0.100 on the WAN side. SSH traffic uses TCP port 22. For Kali to reach machines behind the firewall (DMZ and LAN), the firewall must pass SSH from the WAN to those subnets, or at minimum from 172.31.0.100.

NOTE: The current temp rules (allow-all on WAN) are why SSH works right now. When you replace them with proper rules, you MUST add an explicit SSH allow rule FIRST, before deleting the temp rules. If you delete the temp rules without adding SSH, you will lock yourself out of every machine behind the firewall.

# Current State Summary

I ran a full read-only audit of OPNsense and AD/DNS on March 24. Here is what exists right now and what is broken or missing. The interfaces and IPs are correct, so you do not need to touch those.

| Item | Status | Details |
| --- | --- | --- |
| Interface assignments (WAN/LAN/DMZ) | DONE | vtnet0=WAN, vtnet1=OPT1(DMZ), vtnet2=LAN. IPs correct. |
| OPT1 renamed to DMZ | MISSING | Still shows as OPT1. Cosmetic but fix it for clarity. |
| WAN gateway (172.31.0.1) | DONE | WAN_GW configured and working. |
| LAN gateway (ROUTERLAN_GW) | MISSING | Not in GUI. Route works live but may not survive reboot. |
| Static route to 192.168.1.0/24 | NEEDS FIX | Works in live table but not in persistent config. |
| Outbound NAT | DONE | Hybrid mode. Rules for DMZ, transition, and LAN subnets. |
| Port Forward NAT (WAN → WebServer) | MISSING | No port forward for 80/443 to 10.0.1.100. |
| WAN firewall rules | BROKEN | Two temp allow-all rules. Must be replaced. |
| LAN firewall rules | PARTIAL | Default allow-all works but needs proper scoping. DNS rule has wrong dest. |
| DMZ firewall rules | PARTIAL | DNS outbound exists but too broad. Missing LDAP rule. |
| System DNS | DONE | Points to 192.168.1.5 (AD/DNS). |
| VLAN on vtnet2 | NEEDS FIX | Unknown vlan01 (tag 10). Probably delete it. |
| DHCP on LAN | NEEDS FIX | Range 192.168.1.100–199 overlaps static IPs. Disable or fix. |

# Step-by-Step Fix Order

WARNING: Do these in order. Do not skip ahead. Each step has a test to confirm it works before moving on. If a test fails, stop and troubleshoot before continuing.

## Task 1: Rename OPT1 to DMZ

This is cosmetic but makes every rule easier to read going forward.

Step 1: Log into the OPNsense web GUI at https://192.168.2.1 (from a LAN-side machine) or https://172.31.0.2 (from Kali, works because of the temp rules).

Step 2: Go to Interfaces → [OPT1].

Step 3: Change the Description field from OPT1 to DMZ.

Step 4: Make sure Enable Interface is checked.

Step 5: Click Save, then Apply Changes.

Test: The sidebar should now show WAN, LAN, DMZ instead of WAN, LAN, OPT1.

## Task 2: Create ROUTERLAN_GW Gateway and Static Route

The route to the LAN subnet (192.168.1.0/24) works right now but is not saved in the OPNsense config. It will disappear on reboot. You need to make it persistent.

Step 1: Go to System → Gateways → Configuration. Click Add.

Name:        ROUTERLAN_GW

Interface:   LAN

IP Address:  192.168.2.2

Description: Router-LAN gateway

Step 2: Save the gateway.

Step 3: Go to System → Routes → Configuration. Click Add.

Network:     192.168.1.0/24

Gateway:     ROUTERLAN_GW - 192.168.2.2

Description: LAN subnet via Router-LAN

Step 4: Save, then Apply Changes.

Test: From the OPNsense shell or Diagnostics → Ping, ping 192.168.1.5 (AD/DNS). It should respond. Also confirm the route still shows in Diagnostics → Routes.

## Task 3: Add SSH Allow Rule on WAN (CRITICAL)

WARNING: This rule must be created and saved BEFORE you delete the temp allow-all rules in Task 4. If you do Task 4 first, you will lose SSH access to all machines behind the firewall.

This rule lets Kali (172.31.0.100) SSH through the firewall to reach machines in the DMZ and LAN.

Step 1: Go to Firewall → Rules → WAN. Click Add (the down arrow, to add at top).

Action:       Pass

Interface:    WAN

Protocol:     TCP

Source:       Single host — 172.31.0.100

Destination:  any

Dest Port:    22 (SSH)

Description:  Allow Kali SSH to all zones

Step 2: Save, then Apply Changes.

Test: SSH from Kali to OPNsense (172.31.0.2), then to AD/DNS (192.168.1.5), then to WebServer (10.0.1.100). All three should still work. Do NOT proceed to Task 4 until you confirm this.

NOTE: If SSH breaks after adding this rule, the rule might not have been applied. Check Firewall → Rules → WAN and make sure the new SSH rule is above the temp rules. Apply changes again.

## Task 4: Replace WAN Temp Rules with Proper Rules

Now that SSH is explicitly allowed, you can safely remove the temp rules and add the real ones.

Step 1: Go to Firewall → Rules → WAN.

Step 2: Delete both rules labeled "TEMP allow all WAN in" and "Temp allow all WAN in".

Step 3: Add the following rules (in this order, top to bottom):

Rule A — SSH from Kali (already created in Task 3)

This should already be there from Task 3. Just verify it exists.

Rule B — External HTTP/HTTPS to WebServer

Action:       Pass

Interface:    WAN

Protocol:     TCP

Source:       any

Destination:  10.0.1.100 (WebServer)

Dest Port:    80, 443

Description:  Allow external access to Rent a Dog website

Rule C — Block WAN to LAN

Action:       Block

Interface:    WAN

Protocol:     any

Source:       any

Destination:  192.168.1.0/24

Description:  BLOCK all direct WAN to LAN traffic

Step 4: Save all rules, then Apply Changes.

Test: From Kali, run these checks:

ssh root@172.31.0.2 -o HostKeyAlgorithms=+ssh-rsa     # OPNsense — should work

ssh Administrator@192.168.1.5                          # AD/DNS — should work (SSH rule)

ssh Administrator@10.0.1.100                           # WebServer — should work (SSH rule)

curl http://10.0.1.100                                 # WebServer HTTP — should work (Rule B)

ping 192.168.1.5                                       # ICMP to LAN — may fail (expected)

WARNING: If SSH to AD/DNS or WebServer stops working after this, add the SSH rule back or re-add a temp allow-all until you figure out what went wrong. Do not leave SSH broken.

## Task 5: Clean Up LAN Rules

The LAN side has a couple of broken rules and some duplicates. For now, keep the default allow-all rule so nothing breaks. Just fix the obviously wrong ones.

Step 1: Go to Firewall → Rules → LAN.

Step 2: Delete the rule labeled "LAN to DMZ DNS" (it points to 10.0.1.100 which is wrong — DNS is at 192.168.1.5 and that is on the LAN side anyway, so no cross-zone rule is needed).

Step 3: Edit the rule "LAN to DMZ HTTP" — change the destination port from just 80 to 80, 443 (add HTTPS).

Step 4: Delete the duplicate rule "Lan traffic out" (it duplicates the default allow-all).

Step 5: Leave the default "Default allow LAN to any rule" in place for now. We can tighten this later during hardening.

Step 6: Save and Apply Changes.

Test: From AD/DNS (192.168.1.5), open a browser or use curl to test: curl http://10.0.1.100 (should load the web page). SSH from Kali to LAN machines should still work.

## Task 6: Fix DMZ Rules

Step 1: Go to Firewall → Rules → DMZ (it may still say OPT1 if Task 1 was skipped).

Step 2: Edit the rule "DNS Outbound" — change the source from 10.0.1.100 to 10.0.1.0/24 (whole DMZ net), and change the destination from any to 192.168.1.5 (AD/DNS). Keep port 53 TCP/UDP.

Step 3: Add a new rule for LDAP authentication:

Action:       Pass

Interface:    DMZ

Protocol:     TCP

Source:       10.0.1.0/24 (DMZ net)

Destination:  192.168.1.5 (AD/DNS)

Dest Port:    389, 636

Description:  Allow DMZ to authenticate against AD/LDAP

Step 4: Save and Apply Changes.

Test: From WebServer (10.0.1.100), try: nslookup nestlerteam6.local 192.168.1.5 — should resolve. Also test from Database (10.0.1.200) if possible.

## Task 7: Create Port Forward NAT

Right now there is no port forward, so external traffic to the firewall WAN IP on ports 80/443 does not get redirected to the WebServer. This is needed for the Rent a Dog website to be accessible from outside.

Step 1: Go to Firewall → NAT → Port Forward. Click Add.

Interface:        WAN

Protocol:         TCP

Destination:      WAN address

Dest Port Range:  HTTP (80)

Redirect Target:  10.0.1.100

Redirect Port:    HTTP (80)

Description:      Port forward HTTP to WebServer

Step 2: Add a second rule for HTTPS:

Interface:        WAN

Protocol:         TCP

Destination:      WAN address

Dest Port Range:  HTTPS (443)

Redirect Target:  10.0.1.100

Redirect Port:    HTTPS (443)

Description:      Port forward HTTPS to WebServer

Step 3: Save and Apply Changes.

NOTE: OPNsense may auto-create associated firewall rules on WAN when you add port forwards. If it does, you can remove the manual Rule B from Task 4 since the auto-rule covers it.

Test: From Kali, run: curl http://172.31.0.2 — this should return the WebServer page (traffic hits the firewall WAN IP and gets forwarded to 10.0.1.100).

## Task 8: Quick Cleanup Items

These are low-priority but good to knock out while you are in the GUI.

- Disable DHCP on LAN: Go to Services → DHCPv4 → [LAN] and uncheck Enable. All our machines use static IPs and the DHCP range (192.168.1.100–199) overlaps with Client-WIN11 at .100.

- Investigate vlan01: Go to Interfaces → Other Types → VLAN. There is a VLAN tag 10 on vtnet2 that nobody remembers creating. If it is not needed, delete it.

- Change SNMP community: Go to Services → SNMP. The community string is "public" (default). Either change it or disable SNMP if we are not using it.

# Post-Fix Verification Checklist

After completing all tasks, run through this checklist from Kali (172.31.0.100) to confirm everything works. Check off each one.

| Test | Expected Result | Pass? |
| --- | --- | --- |
| ssh root@172.31.0.2 -o HostKeyAlgorithms=+ssh-rsa | OPNsense shell |  |
| ssh Administrator@192.168.1.5 | AD/DNS PowerShell |  |
| ssh Administrator@10.0.1.100 | WebServer shell |  |
| ssh playerone@10.0.1.200 | Database shell |  |
| ssh vyos@192.168.1.1 -o HostKeyAlgorithms=+ssh-rsa | Router-LAN shell |  |
| ssh Administrator@192.168.1.100 | Client-WIN11 shell |  |
| ssh playerone@192.168.1.51 | Client-LINUX shell |  |
| curl http://172.31.0.2 (port forward test) | WebServer page HTML |  |
| curl http://10.0.1.100 (direct DMZ test) | WebServer page HTML |  |
| nslookup nestlerteam6.local 192.168.1.5 (from Kali) | Resolves to 192.168.1.5 |  |

NOTE: If any SSH connection fails, immediately check Firewall → Rules → WAN and make sure the Kali SSH rule (Task 3) is at the top and enabled. If everything is broken, add a temporary allow-all rule again, test SSH, then work backwards to find which rule caused the problem.

Questions? Text James. Do not change any interface IPs, NAT outbound rules, or SSH configurations. This doc only covers firewall rules, routing, and port forwarding.